Safety Instruction

- Users are required to read the operation manual completely and carefully before installation or operation.

- The product should be installed and pre-operated by well-trained persons. All power supplies must be turned off during the installation work, remember not to operate with power on.

- All the instruction marked with sign  must be observed or executed; otherwise, bodily injuries might occur.

- For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection.

- When connecting the power cord, it must be determined that the operating voltage conforms to the rated voltage value specified in the product identification.

- Don＇t operate in direct sun light，outdoors area and where the room temperature is over 45°C or below 0°C.

- Please avoid operating near the heater at dew area or at the humidity below 10% or above 90%.

- Don＇t operate in area with heavy dust, corrosive substance or volatile gas.

- Avoid power cord being applied by heavy objects or excessive force, or over bend.

- The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

- All the moving portions must be prevented to be exposed by the parts provided.

- Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

- Turn off the power before the following operation:

1. Connecting or disconnecting any connectors on the control box or motor.

2. Threading needle.

3. Raising the machine head.

4. Repairing or doing any mechanical adjustment.

5. Machines idling.

- Repairs and high level maintenance work should only be carried out by electronic technicians with appropriate training.

- All the spare parts for repair must be provided or approved by the manufacturer.

- Don’t use any objects or force to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail

Any trouble found within warranty period under normal operation, it will be repaired free of charge. However, maintenance cost will be charged in the following cases even if within warranty period:

1. Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

2. Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

3. Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency

* We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this product can be damaged due to external magnetic interference and electronic static or noise or unstable power source more than expected; therefore the grounding system of operate area must guarantee the good earth and it’s also recommended to install a failsafe device (Such as residual current breaker).

1 Button displays and operating instructions

1.1 Button description

| Name | Button | Indicate |
| --- | --- | --- |
| Start back-tacking selection |  | If press, switch AB start back-tacking → ABAB start back-tacking → function off → B start back-tacking successively. |
| End back-tacking selection |  | If press, switch CD end back-tacking → CDCD end back-tacking → function off → C end back-tacking successively. |
| Consecutive reverse sewing |  | If press, set to consecutive reverse sewing mode. |
| Free sewing |  | If press, set to free sewing mode. |
| Constant stitch sewing |  | If press, set to constant stitch sewing mode. |
| Multi-segment sewing |  | If press, set to multi- segment sewing mode (switch to four-segment sewing, seven-segment sewing, eight-segment sewing, and fifteen-segment sewing in sequence). |
| Auto function |  | (Only valid for constant stitch sewing) When the function is turned on. The foot pedal is triggered to automatically go through the set sewing process. |
| Thread trimming setting |  | If press, set used or cancelled thread trimming function. |
| Thread wiping setting |  | If press, set used or cancelled thread wiping function. |
| Presser foot lifting setting |  | If press, switch function off → automatic presser foot lifting after trimming → automatic presser foot lifting when pause → full function successively. |
| Needle stop position selection |  | If press, the needle stop position after shift the sewing mode (upper position / lower position). |
| Soft start setting |  | If press, set used or cancelled soft start function. |
| Darning stitch setting |  | When the sewing stops in the middle, if press, make needle lift or darn stitch forward. |
| Function parameter editing |  | If press, enter or exit the parameter setting interface. |
| Setting parameter check and save |  | For the content of the selected parameters check and save: after select parameter press this button to check and modify operation, after modified parameter value press this button to exit and save the parameter. |
| Parameter increasing |  | If press, the parameter will be increased. |
| Parameter decreasing |  | If press, the parameter will be decreased. |
| Speed increasing |  | If press, the speed will be increased. |
| Speed decreasing |  | If press, the speed will be decreased. |

1.2 Auxiliary function

1.2.1 Advanced parameter

Press and hold  to turn on the power and enter the advanced parameter interface.

1.2.2 Factory default setting

In the parameter value interface, press and hold  for 3 seconds to save the current parameter as the factory default value, and “SAVE” will be displayed when the saving is successful.

2 User parameter

| No. | Items | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum sewing speed (rpm) | 100-P68 | 3700 |  |
| P02 | Acceleration curve adjustment (%) | 10-100 | 80 |  |
| P03 | Needle stop position selection | UP/DN | DN | UP: Upper needle stop position DN: Lower needle stop position |
| P04 | Start back-tacking speed (rpm) | 200-3200 | 1800 |  |
| P05 | End back-tacking speed (rpm ) | 200-3200 | 1800 |  |
| P06 | Bar-tacking speed (rpm) | 200-3200 | 1800 |  |
| P07 | Soft start speed (rpm ) | 200-1500 | 400 |  |
| P08 | Stitch numbers for soft start | 1-15 | 2 |  |
| P09 | Automatic constant-stitch sewing speed (rpm) | 200-4000 | 3700 |  |
| P10 | Automatic end back-tacking sewing after constant-stitch sewing | ON/OFF | ON | ON: After executing the constant-stitch sewing, the back-tacking sewing will be executed automatically. In any sewing mode, mending stitch function cannot be used. OFF: After executing the last constant-stitch sewing, the back-tacking sewing function will not be automatically executed, and the front step or full back step must be performed again. |
| P11 | Manual back-tacking mode selection | J/B | J | J: JUKI mode (It will execute back-tacking during sewing or when it stops halfway.) B: BROTHER mode (It will execute back-tacking during sewing, and it will mend needle when it stops halfway.) |
| P12 | Start back-tacking running mode selection | 0-2 | 1 | 0: Controlled by foot pedal, can be stopped and started at will 1: Touch the foot pedal to automatically execute back-tacking action 2: Pause, pattern sewing mode |
| P13 | Ending mode of start back-tacking | CON/STP | CON | CON: Start back-tacking is completed automatically continued for next action. STP: After the number of stitches is completed, stop automatically. |
| P14 | Soft start function selection | ON/OFF | OFF |  |
| P15 | Mending stitch mode | 0-4 | 2 | 0: Half stitch; 1: One stitch; 2: Continuous half stitch; 3: Continuous one stitch; 4: Continuous one stitch, quick stop. |
| P16 | Speed limit of manual back-tacking | 0-3200 | 0 | The function is disabled when the value is 0. |
| P17 | Auto count selection | 0-50 | 1 | 0: P41 item counter not count automatically 1-50: Counter thread trimming times |
| P18 | Start back-tacking compensation 1 | 0-200 | 131 | Stitch compensation for start back-tacking A section, 0~200 action gradually delay; The large value, the longer of the A section last stitch, and the shorter of the B section first stitch. |
| P19 | Start back-tacking compensation 2 | 0-200 | 158 | Stitch compensation for start back-tacking A section, 0~200 action gradually delay; The large value, the longer of the B section last stitch. |
| P21 | The position of the pedal for accelerating | 30-1000 | 520 |  |
| P22 | The position of the pedal for the stop | 30-1000 | 420 |  |
| P23 | The position of the pedal for presser foot lifting | 30-1000 | 270 |  |
| P24 | The position of the pedal for thread trimming | 30-1000 | 130 |  |
| P25 | End back-tacking compensation 3 | 0-200 | 131 | Stitch compensation for end back-tacking C section, 0~200 action gradually delay; The large value, the shorter of the C section first stitch. |
| P26 | End back-tacking compensation 4 | 0-200 | 158 | Stitch compensation for end back-tacking D section,0~200 action gradually delay; the large value, the longer of the C section last stitch, and the shorter of the D section first stitch. |
| P27-N01 | Lock screen time (s) | 0-300 | 0 | The function is disabled when the value is 0. |
| P27-N06 | Automatic piece counting selection | 0-50 | 0 |  |
| P27-N12 | Boot counter interface selection | 0-1 | 0 | 0: OFF 1: ON |
| P27-N13 | Automatic piece counting mode selection | 0-1 | 0 | 0: Incremental piecework mode 1: Diminishing piecework mode |
| P28 | Bar-tacking running mode selection | 0-2 | 1 | 0: Controlled by foot pedal, can be stopped and started at will. 1: Touch the foot pedal to automatically execute back-tacking action. 2: Pause, pattern sewing mode. |
| P29 | The stopping strength of thread trimming | 1-45 | 20 |  |
| P32 | Bar-tacking compensation 5 | 0-200 | 131 | Stitch compensation for bar-tacking A (C) section, 0~200 action gradually delay; the large value, the longer of the A (C) section last stitch; the shorter of the B (D) section first stitch. |
| P33 | Bar-tacking compensation 6 | 0-200 | 158 | Stitch compensation for bar-tacking B (D) section, 0~200 action gradually delay; the large value, the longer of the B (D) section last stitch; the shorter of the C section first stitch. |
| P34 | Constant-stitch sewing running mode selection | A/M | A | A: Touch the foot pedal to automatically execute constant-stitch sewing action M: Controlled by foot pedal, can be stopped and started at will |
| P35 | Thread tension releasing function setting when presser foot lifting action | 0-2 | 0 | 0: OFF 1: Thread tension releasing output function ON when presser foot lifting, thread tension releasing output function OFF when stopover 2: Full function |
| P36 | Thread tension releasing function selection | 0-5 | 3 | 0: OFF 1: The strength of thread tension releasing gradually increases |
| P37 | Thread wiping function / Thread clamping function selection | 0-11 | 8 | 0 : OFF 1: Thread wiping function. 2-11: Thread clamping Function, the larger the value, the stronger the strength of automatic thread clamping. |
| P38 | Automatic thread trimming function selection | ON/OFF | ON |  |
| P39 | Automatic presser foot lifting when pause function selection | UP/DN | DN |  |
| P40 | Automatic presser foot lifting after trimming function selection | UP/DN | DN |  |
| P41 | Thread trimming counter display |  |  | Display the quantity of finished sewing piece. Long-press “-” key to clear the count |
| P42-N01 | The control system version number |  | 12 |  |
| P42-N02 | The panel version number |  | 1239 |  |
| P42-N03 | Speed |  |  |  |
| P42-N04 | The pedal AD |  |  |  |
| P42-N05 | The mechanical angle(upper position) |  |  |  |
| P42-N06 | The mechanical angle(lower position) |  |  |  |
| P42-N07 | Bus bar voltage AD |  |  |  |
| P42-N08 | Zero point of the encoder |  |  |  |
| P42-N09 | Version serial number |  |  |  |
| P42-N10 | Thread trimming position sensing AD value |  |  |  |
| P42-N12 | Status information |  |  |  |
| P43 | Motor rotation direction setting | CCW/CW | CCW | CW: Clockwise CCW: Counter clock wise |
| P44 | Brake strength during stopover | 1-60 | 16 |  |
| P45 | Back-tacking operation duty cycle (%) | 1-60 | 30 | Back-tacking operate in duty cycle to save electricity and protect the electromagnet from over-heat. |
| P46 | After trimming, needle lifting function when main shaft motor rotates in opposite direction | ON/OFF | OFF |  |
| P47 | After trimming, needle lifting angle adjustment when main shaft motor rotates in opposite direction | 50-300 | 160 | Start from the upper needle position and adjust the angle of the needle lift in reverse operation after trimming. |
| P48 | The minimum speed (positioning speed) (rpm ) | 100-500 | 210 | Adjust the minimum speed. |
| P49 | Thread trimming speed (rpm) | 100-500 | 300 | Adjust thread trimming speed. |
| P50 | Presser foot operation time to full output (ms) | 10-990 | 150 |  |
| P51 | Presser foot operation duty cycle (%) | 1-60 | 40 | Presser foot lifter operate in duty cycle to save electricity and protect the electromagnet from over-heat. |
| P52 | Delay the start of the motor to protect the lowering time of presser foot (ms) | 10-990 | 120 | Delay the start time, with automatic presser foot down. |
| P53 | Half back pedaling lifting presser foot function to cancel | ON/OFF | OFF | ON: Half back pedaling with lifting presser foot OFF: Half back pedaling without lifting presser foot |
| P54 | Thread trimming action time (ms) | 10-990 | 200 |  |
| P55 | Thread wiping action time (ms) | 10-990 | 30 |  |
| P56 | Power on and positioning | 0-2 | 1 | 0: Always not to find the upper position 1: Always to find the upper position 2: If the motor is already in the uppere position, no longer find the upper position |
| P57 | Presser foot solenoid protection time (s) | 1-60 | 5 | Forced shutdown at the end of setting time. |
| P58 | Upper position adjustment | 0-2399 |  | Upper position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P59 | Lower position adjustment | 0-2399 |  | Lower position adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P60 | Testing speed (rpm ) | 100-P68 | 3500 |  |
| P61 | Testing A | ON/OFF | OFF | Continuous running testing. |
| P62 | Testing B | ON/OFF | OFF | Start and stop testing with all functions. |
| P63 | Testing C | ON/OFF | OFF | Start and stop testing without all function. |
| P64 | Test running time | 1-250 | 30 |  |
| P65 | Test stopping time | 1-250 | 10 |  |
| P66 | Machine protection switch selection | 0-1 | 1 | 0: OFF 1: ON |
| P68 | Maximum speed limit | 100-5000 | 3700 |  |
| P69 | Motor type selection | 0-50 | 0 | 0: 600 CPR, 6 slots, level 4 1: 360 CPR, 6 slots, level 4 2: 600 CPR, 6 slots, level 4 (large size rotating hook) 3: 360 CPR, 6 slots, level 4 (large size rotating hook) |
| P70 | Model selection |  | 17 |  |
| P71 | Presser foot lowering buffer duty cycle (%) | 0-4 | 0 |  |
| P72 | Upper position quick adjustment | 0-2399 |  | Adjust upper position, the displayed value will change with the position of the handwheel, press "S" key to save the current position (value) as upper position. |
| P73 | Lower position quick adjustment | 0-2399 |  | Adjust lower position, the displayed value will change with the position of the handwheel, press "S" key to save the current position (value) as lower position. |
| P76 | Back-tacking full output time (ms) | 10-990 | 200 |  |
| P77 | Opportunity point of back-tacking for end back-tacking in high speed in free sewing mode (ms) | 20-350 | 160 |  |
| P78 | The starting angle of thread clamping | 0-359 | 100 |  |
| P79 | The stopping angle of thread clamping | 0-359 | 270 |  |
| P80 | Trimming engage angle | 0-359 | 18 | Trimming engage angle setting (down position defined as 0°)) |
| P83 | Stopping strength after trimming | 10-100 | 20 |  |
| P84 | Trimming full output time (ms) | 10-990 | 60 |  |
| P85 | Periodic signal of trimming output (*10%) | 1-10 | 6 |  |
| P87 | The delayed return time of thread wiping | 10-990 | 50 |  |
| P88 | The stopping distance during machine pause | 10-100 | 40 |  |
| P89 | AC overvoltage setting | 500-1023 | 880 |  |
| P92 | Electrical angle correction of motor |  | 160 | Reading the initial Angle of encoder, the factory default was set, please do not change the values (parameter value cannot be changed manually, random change it will result the control box and motor abnormal or damaged). |
| P93 | The delayed time of the half back pedaling function (ms) | 10-900 | 200 |  |
| P94 | Peak value of maximum current | 0-20 | 10 |  |
| P95 | Low servo | 1-800 | 300 |  |
| P96 | Peak value of blocking current | 0-20 | 10 |  |
| P97 | Blocking protection time (s) | 0-10 | 5 |  |
| P98 | Acceleration after thread trimming | 1-800 | 200 |  |
| P100 | Presser foot sensing protection time | 0-250 | 5 |  |
| P101 | The starting angle of thread tension releasing | 1-359 | 30 | Starting angle of thread tension releasing (The lower position is calculated as 0°). |
| P102 | The stopping angle of thread tension releasing | 1-359 | 180 | Stopping angle of thread tension releasing (The lower position is calculated as 0° and needs to be greater than the value of the P101 parameter). |
| P110 | Trimming back time (ms) | 60-990 | 60 |  |
| P113 | Full output action duty cycle of presser foot (%) | 50-100 | 100 |  |
| P114 | Counter mode selection for number of stitches | 0-4 | 0 | 0: OFF 1: Incremental cycle counting 2: Decremental cycle counting 3: Incremental counting, alarm after counting full, need to be cleared by the key to start re-counting. 4: Decremental counting, alarm after counting full, need to be cleared by the key to start re-counting. |
| P117 | The delayed time of press foot lowering buffer (ms) | 0-9999 | 8 |  |
| P119 | Electromagnet overcurrent protection selection | 0-1 | 1 | 0: No alarm, turn off the output. 1: Alarm and shutdown. |
| P120 | Oil level alarm detection | 0-1 | 0 | 0: OFF 1: ON |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

3 Error code list

| Error Code | Problem description | Solutions |
| --- | --- | --- |
| E01 | High voltage | Turn off the machine, check the power supply voltage (Or whether exceed the rated voltage of use). |
| E02 | Low voltage | Turn off the machine, check the power supply voltage (Or whether under the rated voltage of use). |
| E03 | CPU communication abnormal | Turn off the machine, please check if the operation panel in bad connection or is too loose, to get it right and turn on. |
| E05 | Pedal signal abnormal | Turn off the machine, please check if the pedal in bad connection or is too loose, to get it right and turn on. |
| E07 | Main shaft motor locked-rotor | Turn the machine handwheel, check if it is stuck. If it is stuck, you have to rule out the mechanical failure. If turn normal, check if the encoder and motor in bad connection or is too loose. If yes, please correct. If connection well, check if the power supply voltage is abnormal or sewing speed is too high. If yes, please adjust. |
| E08 | Manual back-tacking operation more than 15 seconds | Back-tacking electromagnet response time is too long, you have only to restart the machine. If there are still E-08 after restart the machine, check if the manual back-tacking switch is damaged. |
| E10 | Electromagnet overcurrent protection | Turn off the system power and check whether the electromagnet (solenoid valve) is damaged or short-circuited. |
| E09 E11 | The positioning signal is abnormal | Turn off the machine, please check if the encoder in bad connection or is too loose, to get it right and turn on. |
| E14 | Encoder signal is abnormal | Turn off the machine, please check if the encoder in bad connection or is too loose, to get it right and turn on. |
| E15 | Main shaft motor drive overcurrent | Turn off the machine, then turn on again. |
| E17 | Machine overturned | Turn off the machine, check if the machine is turn over, or the mercury switch is shifted or damaged. |
| E20 | Main shaft motor failed to start (Electrical angle error) | Turn off the machine, please check if the encoder and motor in bad connection or is too loose, to get it right and turn on. |

4 Port diagram

1. Trimming electromagnet: 1, 8 (+32V)

2. Thread wiping electromagnet: 2, 9 (+32V)

3. LED Light: 4 (DGND), 11 (+5V)

4. Back-tacking button: 5 (signal)

5. Back-tacking electromagnet: 6, 13 (+32V)

6. Manual darning stitch button: 7 (signal)