Safety Instruction

- Users are required to read the operation manual completely and carefully before installation or operation.

- The product should be installed and pre-operated by well-trained persons. All power supplies must be turned off during the installation work, remember not to operate with power on.

- All the instruction marked with sign  must be observed or executed; otherwise, bodily injuries might occur.

- For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection.

- When connecting the power cord, it must be determined that the operating voltage conforms to the rated voltage value specified in the product identification.

- Don＇t operate in direct sun light，outdoors area and where the room temperature is over 45°C or below 0°C.

- Please avoid operating near the heater at dew area or at the humidity below 10% or above 90%.

- Don＇t operate in area with heavy dust, corrosive substance or volatile gas.

- Avoid power cord being applied by heavy objects or excessive force, or over bend.

- The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

- All the moving portions must be prevented to be exposed by the parts provided.

- Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

- Turn off the power before the following operation:

1. Connecting or disconnecting any connectors on the control box or motor.

2. Threading needle.

3. Raising the machine head.

4. Repairing or doing any mechanical adjustment.

5. Machines idling.

- Repairs and high level maintenance work should only be carried out by electronic technicians with appropriate training.

- All the spare parts for repair must be provided or approved by the manufacturer.

- Don’t use any objects or force to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail

Any trouble found within warranty period under normal operation, it will be repaired free of charge. However, maintenance cost will be charged in the following cases even if within warranty period:

1. Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

2. Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

3. Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency

* We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this product can be damaged due to external magnetic interference and electronic static or noise or unstable power source more than expected; therefore the grounding system of operate area must guarantee the good earth and it’s also recommended to install a failsafe device (Such as residual current breaker).

1 Main interface instructions

| Name | Key | Indicate |
| --- | --- | --- |
| Setting |  | If press, switch to setting interface. |
| Lock screen |  | If press, switch between lock screen or unlock. |
| Thread trimming |  | If press, set to use or cancel thread trimming function. If long-press, switch to thread trimming setting interface. |
| Needle stop position selection |  | If press, the position of needle after sewing is switched (upper position / lower position). If long-press, switch to needle stop position setting interface. |
| Thread hooking |  | If press, set to use or cancel thread hooking function. If long-press, switch to thread hooking setting interface. |
| Presser foot |  | If press, switch presser foot lifting when pause → presser foot lifting after trimming → full function →function off successively. |
| Presser foot of half back pedaling |  | If press, set to use or cancel presser foot function. If long-press, switch to presser foot setting interface. |
| Soft start |  | If press, set to use or cancel soft start function. If long-press, switch to soft start setting interface. |
| Counting of thread trimming |  | If double-click, the counting is cleared. |
| Speed |  | If press, this data is selected and can be adjusted. |
| Stitch length |  | If press, this data is selected and can be adjusted. |

2 Setting interface instructions

Press in the main interface to enter the setting interface.

| No. | Catalogue |  | Description |
| --- | --- | --- | --- |
| 1 | System setting | Main shaft motor origin correction |  |
|  |  | Stepping motor origin correction |  |
|  |  | Upper needle position setting |  |
|  |  | Lower needle position setting |  |
|  |  | Language |  |
|  |  | Backlight brightness |  |
|  |  | Pedal stroke setting |  |
|  |  | Running-in setting |  |
|  |  | Password setting |  |
|  |  | Reset | Press to restore the latch value; long-press to restore the factory value, and reboot to take effect after restoration. |
| 2 | Monitoring information | Version number |  |
|  |  | Real-time speed |  |
|  |  | Upper needle position |  |
|  |  | lower needle position |  |
|  |  | Pedal AD |  |
|  |  | Busbar voltage |  |
|  |  | Main shaft motor direction |  |
|  |  | Model | Viewable only, not changeable. |
|  |  | Sensor 1 |  |
|  |  | Sensor 2 |  |
|  |  | Number of garments |  |
|  |  | Counting mode |  |
|  |  | Counting ratio |  |
|  |  | System parameter value |  |
| 3 | Parameter setting |  |  |
| 4 | Function testing | Thread wiping testing |  |
|  |  | Thread trimming testing |  |
|  |  | Edge binding testing |  |
|  |  | Blowing testing |  |
|  |  | Presser foot testing |  |

2.3 Parameter setting

| Item | Description | Range | Locking initial value | Chained initial value |
| --- | --- | --- | --- | --- |
| P001 | Maximum speed (rpm) | 100-4000 | 3000 | 3000 |
| P002 | Acceleration curve adjustment (%) | 10-100 | 80 | 80 |
| P003 | Needle stop position selection | 0-1 | 0 | 0 |
| P007 | Soft start speed after second stitch (rpm) | 100-1500 | 1000 | 1000 |
| P008 | The number of stitches for soft start (stitch) | 1-100 | 10 | 10 |
| P014 | Soft start function setting | 0-1 | 1 | 1 |
| P021 | The position of the pedal for acceleration | 30-1000 | 520 | 520 |
| P022 | The position of the pedal for stopping | 30-1000 | 420 | 420 |
| P023 | The position of the pedal for presser foot lifting | 30-1000 | 300 | 300 |
| P024 | The position of the pedal for thread trimming | 30-500 | 180 | 180 |
| P029 | Stopping strength when thread trimming | 1-50 | 20 | 20 |
| P037 | Thread hooking function setting | 0-1 | 1 | 1 |
| P038 | Thread trimming function setting | 0-1 | 1 | 1 |
| P039 | Presser foot lifting setting when pause | 0-1 | 0 | 0 |
| P040 | Presser foot lifting setting after trimming | 0-1 | 0 | 0 |
| P043 | Direction of motor rotation setting | 0-1 | 1 | 1 |
| P044 | Braking strength when pause | 1-50 | 16 | 16 |
| P046 | After trimming, needle lifting function when main shaft motor rotates in opposite direction | 0-1 | 0 | 0 |
| P047 | After trimming, needle lifting angle adjustment when main shaft motor rotates in opposite direction | 0-359 | 40 | 40 |
| P048 | The minimum speed (positioning speed) (rpm) | 100-500 | 200 | 200 |
| P049 | Trimming speed (rpm) | 100-500 | 230 | 230 |
| P050 | The working time of presser foot full output (ms) | 10-990 | 200 | 200 |
| P051 | Periodic signal of presser foot output (%) | 1-60 | 38 | 38 |
| P052 | The motor is delayed to start to protect the lowering time of presser foot (ms) | 10-990 | 120 | 120 |
| P053 | Presser foot lifting function cancel when half back pedaling | 0-2 | 1 | 1 |
| P054 | Chain thread trimming action time (ms) | 10-990 | 150 | 100 |
| P055 | Thread hooking action time (ms) | 10-990 | 100 | 100 |
| P056 | Automatically find the upper position after power-on | 0-1 | 0 | 0 |
| P058 | Upper positioning adjustment | 0-359 | 205 | 205 |
| P059 | Lower positioning adjustment | 0-359 | 15 | 15 |
| P060 | Testing speed | 100-4000 | 3500 | 3500 |
| P062 | Testing B | 0-1 | 0 | 0 |
| P064 | Test running time | 1-50 | 30 | 30 |
| P065 | Test stopping time | 1-50 | 10 | 10 |
| P068 | Maximum speed limit | 100-4000 | 3500 | 3500 |
| P070 | Model selection | 1-50 | 1 | 2 |
| P072 | Upper positioning quick adjustment | 0-359 | 205 | 205 |
| P073 | Lower positioning quick adjustment | 0-359 | 15 | 15 |
| P074 | Tacking stitch length overall compensation | -100~100 | 0 | 0 |
| P080 | Locking trimming engage angle | 1-359 | 50 | 50 |
| P082 | Locking trimming retract angle | 1-359 | 125 | 125 |
| P083 | The stopping distance after trimming | 10-100 | 20 | 20 |
| P086 | Upper and lower positioning distance | 15-345 | 200 | 220 |
| P087 | The delayed return time of thread hooking | 10-990 | 250 | 250 |
| P088 | The stopping distance when pause | 10-100 | 30 | 30 |
| P089 | AC overvoltage value setting | 500-1023 | 980 | 980 |
| P090 | Soft start first stitch speed | 100-1000 | 500 | 500 |
| P091 | Soft start second stitch speed | 100-2000 | 1000 | 1000 |
| P092 | Electrical angle correction of motor | 0-1200 | 155 | 155 |
| P093 | The starting time of half back pedaling (ms) | 10-900 | 250 | 250 |
| P099 | Stitch length of starting tight seam | 0-30 | 8 | 8 |
| P107 | Starting tight seam speed | 100-2000 | 1200 | 1200 |
| P108 | The number of stitches for starting tight seam | 0-12 | 0 | 0 |
| P109 | The delayed time before thread hooking | 5-990 | 5 | 5 |
| P110 | The return time of thread trimming (ms) | 5-990 | 5 | 5 |
| P116 | Duty cycle of blowing | 0-75 | 60 | 60 |
| P131 | Normal stitch length | 0-60 | 50 | 50 |
| P153 | Stitch length of ending tight seam | 0-30 | 8 | 8 |
| P154 | Ending tight seam speed | 100-2000 | 1200 | 1200 |
| P160 | The number of stitches for ending tight seam | 0-12 | 0 | 0 |
| P205 | First stitch speed limit when start sewing (rpm) | 0-1000 | 600 | 600 |
| P206 | Second stitch speed limit when start sewing (rpm) | 0-2000 | 1200 | 1200 |
| P207 | Speed limit switch when start sewing | 0-1 | 1 | 1 |
| P221 | Maximum speed position of pedal | 0-1023 | 820 | 820 |
| P234 | Distance between the entry point and the upper position | 0-1000 | 300 | 300 |
| P235 | Synchronization of cloth puller | 0-1 | 0 | 0 |
| P236 | Running stroke of cloth puller | 1-200 | 118 | 118 |
| P237 | Stitch length ratio of cloth puller | 1-500 | 100 | 100 |
| P238 | Locking trimming protection time | 1-990 | 200 | 200 |
| P239 | Blowing function selection | 0-1 | 1 | 1 |
| P240 | Edge binding setting | 0-1 | 1 | 1 |
| P241 | Running direction setting of cloth puller | 0-1 | 0 | 0 |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

3 Port diagram

14P port diagram

1. Edge binding output: 7 (+32V), 14

2. Thread hooking output: 6 (+32V), 13

3. Thread trimming output: 5 (+32V), 12

4. LED light: 4 (LED+), 11 (LED-)

4P port diagram

Kneeing-control device sensing: 1 (DGND), 4 (IN)

2P (white) port diagram

Presser foot solenoid valve: 1 (S32V), 2 (OUT)

2P (red) port diagram

Blowing solenoid valve: 1 (S32V), 2 (OUT)