Safety Instruction

- Users are required to read the operation manual completely and carefully before installation or operation.

- The product should be installed and pre-operated by well-trained persons. All power supplies must be turned off during the installation work, remember not to operate with power on.

- All the instruction marked with sign  must be observed or executed. Otherwise, bodily injuries might occur.

- For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection.

- When connecting the power cord, it must be determined that the operating voltage conforms to the rated voltage value specified in the product identification.

- Don＇t operate in direct sun light，outdoors area and where the room temperature is over 45°C or below 0°C.

- Please avoid operating near the heater at dew area or at the humidity below 10% or above 90%.

- Don＇t operate in area with heavy dust, corrosive substance or volatile gas.

- Avoid power cord being applied by heavy objects or excessive force, or over bend.

- The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

- All the moving portions must be prevented to be exposed by the parts provided.

- Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

- Turn off the power before the following operation:

1. Connecting or disconnecting any connectors on the control box or motor.

2. Threading needle.

3. Raising the machine head.

4. Repairing or doing any mechanical adjustment.

5. Machines idling.

- Repairs and high level maintenance work should only be carried out by electronic technicians with appropriate training.

- All the spare parts for repair must be provided or approved by the manufacturer.

- Don’t use any objects or force to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail

Any trouble found within warranty period under normal operation, it will be repaired free of charge. However, maintenance cost will be charged in the following cases even if within warranty period:

1. Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

2. Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

3. Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency

* We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this product can be damaged due to external magnetic interference and electronic static or noise or unstable power source more than expected. Therefore the grounding system of operate area must guarantee the good earth and it’s also recommended to install a failsafe device (Such as residual current breaker).

1 Button displays and operating instructions

1.1 Key description

| Name | key | Indicate |
| --- | --- | --- |
| Function key |  | If press, enter or exit function parameter edit mode. If long-press, switch to the password input interface. Enter the correct password, press the S key to confirm, you can enter the advanced parameter setting interface. |
| Enter key |  | For the content of the selected parameters check and save: after select parameter press this key to check and modify operation, after modified parameter value press this key to exit and save the parameter. In the main interface, if long-press, switch to the debugging parameter interface. P72, P92, P100 can be adjusted. |
| Left selection |  | The parameter bit is shifted left. |
| Right selection |  | The parameter bit is shifted right. |
| Parameter increasing |  | If press, the parameter will be increased. If long-press, the parameter will be increased continuously. |
| Parameter decreasing |  | If press, the parameter will be decreased. If long-press, the parameter will be decreased continuously. |
| Speed increasing |  | If press, the speed will be increased. |
| Speed decreasing |  | If press, the speed will be decreased. |
| LED brightness adjustment |  | In the main interface, if press, adjust LED brightness. |
| Soft start setting |  | In the main interface, if press, set used or cancelled soft start function. |
| Presser foot lifting setting |  | In the main interface, if press, set used or cancelled presser foot lifting function. |
| Needle stopping position selection |  | In the main interface, if press, switch between upper and lower needle stop positions. |

1.2 Auxiliary function

1.2.1 Debugging mode

Long press “” in the main interface, switch to the debugging parameter interface. P72, P92, P100 can be adjusted.

2 User parameter

| No. | Items | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum sewing speed (rpm) | 100-9000 | 5000 |  |
| P02 | Acceleration curve adjustment (%) | 10-100 | 80 |  |
| P03 | Needle stopping position selection | UP/DN | UP | UP: Upper stopping position DN: Lower stopping position |
| P04 | Output duty cycle of upper thread wiping electromagnet | 0-100 | 75 |  |
| P05 | Upper thread wiping action time (ms) | 10-990 | 20 |  |
| P06 | Upper thread wiping return time (ms) | 10-990 | 15 |  |
| P07 | Soft start speed (rpm) | 200-1500 | 1000 |  |
| P08 | The number of stitches for soft start (stitch) | 0-99 | 2 |  |
| P14 | Soft start function selection | ON/OFF | OFF |  |
| P15 | LED brightness setting | 0-4 | 3 |  |
| P21 | The position of the pedal for accelerating | 30-1000 | 500 |  |
| P22 | The position of the pedal for the stop | 30-1000 | 420 |  |
| P23 | The position of the pedal for presser foot lifting | 30-1000 | 280 |  |
| P24 | The position of the pedal for thread trimming | 30-1000 | 130 |  |
| P29 | The stopping force when thread trimming | 1-45 | 20 |  |
| P37 | Thread trimming of upper knife output function setting | 0-11 | 1 |  |
| P39 | Automatic presser foot lifting when pause | UP/DN | DN | UP: ON DN: OFF |
| P40 | Automatic presser foot lifting after trimming | UP/DN | DN | UP: ON DN: OFF |
| P41 | Counter display | 0-9999 |  | Display the quantity of finished sewing piece. Long-press “-” key to clear the count. |
| P42-N01 | Main software version |  |  |  |
| P42-N02 | Panel software version |  |  |  |
| P42-N03 | Speed |  |  |  |
| P42-N04 | The pedal AD |  |  |  |
| P42-N05 | The mechanical angle (upper position) |  |  |  |
| P42-N06 | The mechanical angle (lower position) |  |  |  |
| P42-N07 | Busbar voltage AD |  |  |  |
| P42-N25 | View virtual password |  |  |  |
| P43 | Direction of motor rotation | CCW/CW | CW | CW: Clockwise CCW: Counter clockwise |
| P44 | Normal stopping force | 1-45 | 15 |  |
| P48 | The minimum speed (positioning speed) (rpm) | 200-1000 | 500 | Adjust the minimum speed. |
| P50 | Presser foot full output time | 10-990 | 150 |  |
| P51 | Cycle signal of presser foot output | 1-80 | 30 |  |
| P52 | Delay the start of the motor to protect the lowering time of presser foot (ms) | 10-990 | 100 | Delay the start time, with automatic presser foot down. |
| P53 | Presser foot lifting function selection | 0-2 | 1 | 0: OFF 1: Back pedaling and half back pedaling with lifting presser foot 2: Half back pedaling without lifting presser foot, back pedaling with lifting presser foot |
| P54 | Thread trimming of lower knife action time | 0-100 | 10 |  |
| P55 | Thread trimming of upper knife action time (ms) | 10-990 | 20 |  |
| P56 | Power on and positioning | 0-2 | 0 | 0: Always not to find the upper position 1: Always to find the upper position 2: If the motor is already in the upper position, no longer find the upper position |
| P57 | Presser foot protection time (s) | 1-120 | 10 | Forced to close after lifting the presser foot for a holding time. |
| P58 | Upper positioning adjustment | 0-2399 |  | Upper positioning adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P59 | Down positioning adjustment | 0-2399 |  | Down positioning adjustment, the needle will advance stop when the value decreased, the needle will delay stop when the value increased. |
| P60 | Testing speed (rpm) | 100-5500 | 4000 | Setting testing speed. |
| P61 | Testing A | ON/OFF | OFF | Continuous running testing. |
| P62 | Testing B | ON/OFF | OFF | Start and stop testing with all functions. |
| P63 | Testing C | ON/OFF | OFF | Start and stop testing without all function. |
| P64 | Test running time | 1-250 | 50 |  |
| P65 | Test stopping time | 1-250 | 50 |  |
| P67 | Lower thread trimming position protection switch | ON/OFF | ON |  |
| P68 | Maximum speed limit (rpm) | 100-5500 | 5500 |  |
| P69 | Motor type selection | 0-50 | 0 |  |
| P70 | Model selection | 1-45 | 32 |  |
| P71 | Presser foot lowering buffer duty cycle (%) | 0-50 | 0 |  |
| P72 | Quick adjustment of upper positioning | 0-2399 |  | Adjust up needle position, the displayed value will change with the position of the handwheel, press "S" key to save the current position (value) as up needle position. |
| P73 | Quick adjustment of down positioning | 0-2399 |  | Adjust down needle position, the displayed value will change with the position of the handwheel, press "S" key to save the current position (value) as down needle position. |
| P86 | Upper and lower positioning distance | 0-2399 | 1200 |  |
| P87 | The delayed return time for thread trimming of upper knife (ms) | 10-990 | 15 |  |
| P89 | AC overvoltage setting | 500-1023 | 880 |  |
| P92 | Electrical angle correction of motor |  | 160 | Reading the initial Angle of encoder, the factory default was set, please do not change the values (parameter value cannot be changed manually, random change it will result the control box and motor abnormal or damaged). |
| P93 | The delayed time of half back pedaling (ms) | 10-900 | 150 |  |
| P104 | Password setting | 0000-9999 | 2016 |  |
| P108 | Suction function selection | 0-1 | 0 | 0: OFF 1: ON |
| P113 | Presser foot lifting holding phase control type | 0-1 | 0 | 0: Open loop 1: Closed loop |
| P116 | The working time of press foot lowering buffer (ms) | 0-990 | 30 |  |
| P117 | The delayed time of press foot lowering buffer (ms) | 0-990 | 20 |  |
| P119 | Electromagnet overcurrent protection | 0-1 | 1 | 0: No alarm, turn off the output. 1: Alarm and shutdown. |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

3 Error code list

| Error code | Problem description | Solutions |
| --- | --- | --- |
| E01 | High voltage | 1. Whether the grid voltage is higher than AC260V. 2. If it is self-generated power supply, please reduce the generator power. 3. If it still does not work normally, please replace the control box and notify the after-sales service. |
| E02 | Low voltage | 1. Whether to connect to low voltage. 2. Reset. 3. If it still does not work normally, please replace the control box and notify the after-sales service. |
| E03 | CPU communication abnormal | 1. Turn off the system power and check whether the connection of the display screen is loose or disconnected, restart the system after returning it to normal. 2. Turn off the system power, remove the control box and only plug in the power cord to power on, whether alarm E03, if it still alarms E03, replace the control box and notify the after-sales service. |
| E05 | Pedal signal abnormal | 1. Check whether the pedal connector is loose or fall off, and restart the system after returning it to normal. 2. If it still does not work normally, please replace the control box or speed controller and notify the after-sales service. |
| E07 | Main shaft motor locked-rotor | 1. Turn off the power and check whether the handwheel can be turned smoothly (turn the handwheel by hand), if it cannot be turned, please check the machine. 2. Turn off the power, check whether the motor power connector is loose, plug it in and restart it. 3. Check whether the upper needle stop position is correct, if not, please adjust the upper positioning position. 4. If it still does not work normally, please replace the control box or spindle motor and notify the after-sales service. |
| E10 | Electromagnet overcurrent protection | Turn off the system power and check the solenoid (solenoid valve) for damage or short circuit. |
| E09 E11 | The positioning signal of main shaft motor encoder is abnormal | 1. Turn off the system power, check whether main shaft motor encoder connector is loose or fall off, restore it to normal and restart the system. 2. Check whether the motor zero point correction setting is correct; Reset the motor zero point correction; Whether there is oil on the encoder code plate, please clean it if there is any. 3. If it still does not work normally, please replace the control box or main shaft motor and notify the after-sales service. |
| E14 | Main shaft motor encoder signal is abnormal | 1. Turn off the system power, check whether the main shaft motor encoder connector is loose or fall off, restore it to normal and restart the system. 2. Check whether the grating is installed correctly (whether the grating screws are tightened and whether the grating is in the center of the encoder). 3. Check whether there is oil on the encoder code plate, if there is, please clean it up, and restart the system after recovery. 4. If it still does not work normally, please replace the control box or main shaft motor and notify the after-sales service. |
| E15 | Main shaft motor drive overcurrent | 1. Please check whether the motor power cord has bad contact. 2. Please check whether the motor power cord is crushed. 3. Please replace the control box or main shaft motor and notify the after-sales service. |
| E20 | Main shaft motor failed to start | 1. Turn off the system power, check whether main shaft motor power cord connector and encoder connector are loose or fall off, restore them to normal and restart the system. 2. Check whether the motor zero point correction setting is correct, reset the motor zero point correction. 3. If it still does not work normally, please replace the control box or main shaft motor and notify the after-sales service. |
| E94 | The positioning signal of trimming (presser foot) stepping motor encoder is abnormal | 1. Turn off the system power and observe whether trimming (presser foot) stepping motor is stuck. If it is stuck, remove the mechanical failure of the machine first. If it is normal, check whether trimming (presser foot) stepping motor encoder connector is loose or falling off, and whether there is oil on the encoder code plate. If there is, please clean it up, restore it to normal and restart the system. 2. If it still does not work normally, please replace the control box or trimming (presser foot) stepping motor and notify the after-sales service. |
| E95 | Trimming (presser foot) stepping motor encoder signal is abnormal | 1. Turn off the system power, check if the thread trimming (presser foot) stepping motor encoder connector is loose or fall off, restore it to normal and restart the system. 2. If it still does not work normally, please replace the control box or the thread trimming (presser foot) stepping motor and notify the after-sales service. |
| E96 | Trimming (presser foot) stepping motor failed to start | 1. Turn off the power of the system, check whether the thread trimming (presser foot) stepping motor power cord connector and encoder connector are loose or fall off, and restart the system after returning it to normal. 2. If it still does not work normally, please replace the control box or the thread trimming (presser foot) stepping motor and notify the after-sales service. |
| E98 | The stepping motor is locked-rotor when presser foot lifting | 1. Turn off the system power and observe whether the thread trimming (presser foot) stepping motor is stuck. If it is stuck, remove the mechanical failure of the machine first. If it is normal, check whether the thread trimming (presser foot) stepping motor power cord connector and encoder connector are loose or fall off, restore them to normal and restart the system. 2. If it still does not work normally, please replace the control box or the thread trimming (presser foot) stepping motor and notify the after-sales service. |
| E99 | The stepping motor overcurrent when presser foot lifting | 1. Turn off the system power and observe whether the thread trimming (presser foot) stepping motor is stuck. If it is stuck, remove the mechanical failure of the machine first. If it is normal, check whether the thread trimming (presser foot) stepping motor power cord connector and encoder connector are loose or fall off, restore them to normal and restart the system. 2. If it still does not work normally, please replace the control box or the thread trimming (presser foot) stepping motor and notify the after-sales service. |

4 Port diagram

2P function port description

1. LED lights: 1 (DGND), 2 (+5V)