Safety Instruction

- Users are required to read the operation manual completely and carefully before installation or operation.

- The product should be installed and pre-operated by well-trained persons. All power supplies must be turned off during the installation work, remember not to operate with power on.

- All the instruction marked with sign  must be observed or executed. Otherwise, bodily injuries might occur.

- For perfect operation and safety, it is prohibited that using extension cable with multi-outlet for power connection.

- When connecting the power cord, it must be determined that the operating voltage conforms to the rated voltage value specified in the product identification.

- Don＇t operate in direct sun light，outdoors area and where the room temperature is over 45°C or below 0°C.

- Please avoid operating near the heater at dew area or at the humidity below 10% or above 90%.

- Don＇t operate in area with heavy dust, corrosive substance or volatile gas.

- Avoid power cord being applied by heavy objects or excessive force, or over bend.

- The earth wire of power cord must be connected to the system ground of the production plant by proper size of conductions and terminals. This connection should be fixed permanently.

- All the moving portions must be prevented to be exposed by the parts provided.

- Turing on the machine in the first time, operate the sewing machine at low speed and check the correct rotation direction.

- Turn off the power before the following operation:

1. Connecting or disconnecting any connectors on the control box or motor.

2. Threading needle.

3. Raising the machine head.

4. Repairing or doing any mechanical adjustment.

5. Machines idling.

- Repairs and high level maintenance work should only be carried out by electronic technicians with appropriate training.

- All the spare parts for repair must be provided or approved by the manufacturer.

- Don’t use any objects or force to hit or ram the product.

Guarantee Time

Warranty period of this product is 1 year dated from purchasing, or within 2 years from ex-factory date.

Warranty Detail

Any trouble found within warranty period under normal operation, it will be repaired free of charge. However, maintenance cost will be charged in the following cases even if within warranty period:

1. Inappropriate use, including: wrong connecting high voltage, wrong application, disassemble, repair, modification by incompetent personnel, or operation without the precaution, or operation out of its specification range, or inserting other objects or liquids into the product.

2. Damage by fire, Earth quake, lighting, wind, flood, salt corrosive, moisture, abnormal power voltage and any other damage cause by the natural disaster or by the inappropriate environments.

3. Dropping after purchasing or damage in transportation by customer himself or by customer’s shipping agency

* We make our best effort to test and manufacture the product for assuring the quality. However, it is possible that this product can be damaged due to external magnetic interference and electronic static or noise or unstable power source more than expected. Therefore the grounding system of operate area must guarantee the good earth and it’s also recommended to install a failsafe device (Such as residual current breaker).

1 Key displays and operating instructions

  - Key description

| Name | key | Indicate |
| --- | --- | --- |
| Function key |  | If press, enter or exit the user parameter setting interface. If long-press, switch to password input interface. Input correct password and press S to confirm, you can enter the advanced parameter setting interface. |
| Confirm key |  | View and save the content of the selected parameter number: press this key after selecting the parameter number to view and modify the operation; press this key after modifying the parameter value to exit and save the parameter. In the main interface, if long-press, it will switch to the interface of debugging parameter item, and you can adjust P100 item, P92 item and P72 item. |
| Parameter increasing |  | If press, the running speed will be increased. |
| Parameter decreasing |  | If press, the running speed will be decreased. |
| Constant stitch sewing mode |  | In the main interface, if press, set to use or cancel constant stitch sewing mode. In the parameter interface, if press, increase the value of thousands digit. |
| Thread trimming of lower knife |  | In the main interface, if press, set to use or cancel thread trimming of 1ower knife and thread trimming of upper knife function. In the parameter interface, if press, increase the value of hundreds digit. |
| Thread trimming of upper knife |  | In the main interface, if press, set to use or cancel thread trimming of upper knife function. In the parameter interface, if press, increase the value of tens digit. |
| Auto function |  | In the main interface, if press, set to use or cancel auto function. In the parameter interface, if press, increase the value of units digit. |
| Stopping position of needle |  | In the main interface, if press, switch upper or lower position. In the parameter interface, if press, decrease the value of thousands digit. |
| Pedal gear adjustment |  | In the main interface, if press, switch 1 (regular gear) → 2 (slower gear)→ 3 (slow gear)→ 4 (fast gear) successively.. In the parameter interface, if press, decrease the value of hundreds digit. |
| Presser foot lifting when pause |  | In the main interface, if press, set to use or cancel presser foot lifting when pause function. In the parameter interface, if press, decrease the value of tens digit. |
| Presser foot lifting after trimming |  | In the main interface, if press, set to use or cancel presser foot lifting after trimming function. In the parameter interface, if press, decrease the value of units digit. |

1.2 Auxiliary function

1.2.1 Debugging mode

In the main interface, if long-press  , it will switch to the interface of debugging parameter item, and you can adjust P92 item, P72 item and P100 item.

1.2.2 Restore factory settings

In the main interface, long-press  and  to restore factory settings.

1.2.3 User parameter latch

Select the parameter item, press  to enter the parameter value interface, then long-press  to display SAVE, which indicates that the parameter value of the current parameter item has been latched, and the parameter value will not be restored when restoring the factory setting.

2 User parameter

| No. | Items | Range | Default | Description |
| --- | --- | --- | --- | --- |
| P01 | Maximum speed (rpm) | 100-9000 | 3500 |  |
| P02 | Set accelerated curve (%) | 10-100 | 80 | Set the acceleration slope. The greater the slope value, the steeper the speed; the smaller the slope value, the slower the speed. |
| P03 | Needle stop position selection | UP/DN | UP | UP: Upper position DN: Lower position |
| P07 | Soft start speed (rpm) | 200-1500 | 1000 |  |
| P08 | Stitch numbers for soft start | 0-99 | 2 |  |
| P14 | Soft start | ON/OFF | OFF |  |
| P15 | LED light brightness adjustment | 0-4 | 2 |  |
| P21 | The position of the pedal for accelerating | 30-1000 | 500 |  |
| P22 | The position of the pedal for stopping | 30-1000 | 420 |  |
| P23 | The position of the pedal for presser foot lifting | 30-1000 | 280 |  |
| P24 | The position of the pedal for thread trimming | 30-1000 | 130 |  |
| P27-N06 | Automatic piece counting function | 0-50 | 1 | 0: P41 function off 1-50: Counting times of thread trimming setting |
| P27-N07 | Buzzer switch | 0-1 | 1 | 0: OFF 1: ON |
| P29 | Stopping strength after thread trimming | 1-45 | 20 |  |
| P37 | Thread trimming of upper knife output function setting | ON/OFF | ON |  |
| P38 | Thread trimming of lower knife output function setting | ON/OFF | ON |  |
| P39 | Automatic presser foot lifting when pause function selection | UP/DN | DN |  |
| P40 | Automatic presser foot lifting after trimming function selection | UP/DN | DN |  |
| P41 | Counter display | 0-9999 | 0 | In the main interface, long press and at the same time can quickly view the number of counting. |
| P42 | Information display |  |  | N01 The control system version number N02 The panel version number N03 Speed N04 The pedal AD N05 The mechanical angle(upper position) N06 The mechanical angle(lower position) N07 Bus bar voltage AD |
| P43 | Motor rotation direction setting | CCW/CW | CW | CW: Clockwise, reverse rotation CCW: Counter clockwise, forward rotation |
| P44 | Normal stopping strength | 1-45 | 15 |  |
| P48 | The minimum speed (positioning speed) (rpm) | 100-500 | 200 |  |
| P50 | The working time of presser foot full output (ms) | 10-500 | 150 |  |
| P51 | Periodic signal of presser foot output (%) | 1-50 | 26 | Presser foot action with periodic power saving output to avoid electromagnet getting hot. |
| P52 | Delay the start of the motor to protect the lowering time of presser foot (ms) | 10-990 | 110 | Delay the start time, with automatic presser foot down. |
| P53 | Presser foot lifting function selection | 0-2 | 1 | 0: OFF 1: Back pedaling and half back pedaling with lifting presser foot 2: Half back pedaling without lifting presser foot, back pedaling with lifting presser foot |
| P54 | Thread trimming of lower knife action time (ms) | 10-990 | 100 |  |
| P55 | Thread trimming of upper knife action time (ms) | 10-990 | 30 |  |
| P56 | Automatically find the upper position after power-on | 0-1 | 0 | 0: OFF 1: ON |
| P57 | Presser foot lifting protection time (s) | 1-60 | 10 | Forced closure after the presser foot lifting holding time has expired. |
| P60 | Testing speed (r/s) | 100-P68 | 4000 |  |
| P61 | Testing A | ON/OFF | OFF | Continuous running testing. |
| P62 | Testing B | ON/OFF | OFF | Start and stop testing with all functions. |
| P63 | Testing C | ON/OFF | OFF | Start and stop testing without all function. |
| P64 | Testing B and C conduction time | 1-250 | 50 |  |
| P65 | Testing B and C stopping time | 1-250 | 50 |  |
| P67 | Thread trimming of lower knife position protection switch | ON/OFF | ON |  |
| P68 | Maximum speed limit (rpm) | 100-7500 | 4500 |  |
| P70 | Model selection |  | 15 |  |
| P71 | Presser foot releasing buffer duty cycle (%) | 0-50 | 0 |  |
| P86 | Upper and lower positioning distance | 0-2399 | 1200 | Upper and lower positioning distance angle (1 degree for every 4 values). |
| P87 | Return delay time of thread trimming of upper knife (ms) | 10-990 | 30 |  |
| P88 | Thread trimming of upper knife solenoid output duty cycle | 0-100 | 75 |  |
| P89 | AC overvoltage setting | 500-1023 | 880 |  |
| P92 | Electrical angle correction of motor |  | 160 | Reading the initial angle of encoder, the factory default was set, please do not change the values (parameter value cannot be changed manually, random change it will result the control box and motor abnormal or damaged). |
| P93 | Half back pedaling function onset time (ms) | 10-900 | 150 |  |
| P105 | Thread trimming of upper knife port output selection | 0-1 | 0 | 0: Thread trimming of upper knife (thread wiping) 1: Blowing |
| P109 | Thread trimming of lower knife return time (ms) | 5-990 | 60 |  |
| P113 | Presser foot full output cycle signal (%) | 0-100 | 90 |  |
| P114 | Constant stitch sewing switch | ON/OFF | OFF |  |
| P115 | Auto function | ON/OFF | OFF |  |
| P116 | The number of stitches for constant stitch sewing | 0-990 | 100 |  |
| P117 | The buffer delayed time of presser foot releasing (ms) | 0-990 | 20 |  |
| P118 | Tight seam port output type | 0-1 | 0 | 0: Blowing of presser foot 1: Tight seam |
| P119 | Electromagnet overcurrent protection detection switch | 0-1 | 1 | 0: No alarm, shutdown output 1: Alarm and shutdown |
| P120 | Blowing of presser foot switch | 0-1 | 1 |  |
| P122 | Pedal gear adjustment | 1-4 | 4 |  |
| P126 | Upper positioning adjustment of hall components | -359~359 | 90 | The upper positioning can be fine-tuned. |
| P127 | Lower positioning adjustment of hall components | -359~359 | 0 | The lower positioning can be fine-tuned. |
| P130 | Advance angle of tight seam electromagnet movement (degrees) | 10-350 | 75 |  |
| P138 | Blowing duty cycle of presser foot | 1-100 | 80 |  |
| P139 | Blowing movement time of presser foot | 1-1000 | 100 | Parameter value *10 ms |
| P140 | Tight seam mode | 0-3 | 3 | 0: OFF 1: Starting tight seam 2: Ending tight seam 3: Full function |
| P141 | Starting tight seam stitch length | 0-60 | 20 |  |
| P142 | Starting tight seam stitch number | 0-10 | 3 |  |
| P143 | Starting tight seam speed | 200-1500 | 700 |  |
| P144 | Ending tight seam stitch length | 0-60 | 20 |  |
| P145 | Ending tight seam stitch number | 0-10 | 3 |  |
| P146 | Ending tight seam speed | 200-1500 | 700 |  |
| P147 | Output duty cycle of tight seam | 1-100 | 80 |  |
| P161 | Fast discharge time of thread trimming of lower knife electromagnet | 0-9999 | 12 | Increasing the value slows down the thread trimming of lower knife return speed and reduces the return sound. |

Note: the initial value of parameters is for reference only, and the actual value of parameters is subject to the real object.

3 Error code list

| Error Code | Problem description | Solutions |
| --- | --- | --- |
| E01 | High voltage | 1. Whether the grid voltage is higher than AC260V. 2. If it is self-generated power supply, please reduce the generator power. 3. If it still does not work normally, please replace the control box and notify the after-sales service. |
| E02 | Low voltage | 1. Whether to connect to low voltage. 2. Reset. 3. If it still does not work normally, please replace the control box and notify the after-sales service. |
| E03 | CPU communication abnormal | 1. Turn off the system power and check whether the connection of the display screen is loose or disconnected, restart the system after returning it to normal. 2. Turn off the system power, remove the control box and only plug in the power cord to power on, whether alarm E03, if it still alarms E03, replace the control box and notify the after-sales service. |
| E05 | Pedal signal abnormal | 1. Check whether the pedal connector is loose or fall off, and restart the system after returning it to normal. 2. If it still does not work normally, please replace the control box or speed controller and notify the after-sales service. |
| E07 | Main shaft motor locked-rotor | 1. Turn off the power and check whether the handwheel can be turned smoothly (turn the handwheel by hand), if it cannot be turned, please check the machine. 2. Turn off the power, check whether the motor power connector is loose, plug it in and restart it. 3. Check whether the upper needle stop position is correct, if not, please adjust the upper positioning position. 4. If it still does not work normally, please replace the control box or spindle motor and notify the after-sales service. |
| E10 | Electromagnet overcurrent protection | Turn off the system power and check the electromagnet (electromagnet valve) for damage or short circuits. |
| E09 E11 | The positioning signal of main shaft motor encoder is abnormal | 1. Turn off the system power, check whether main shaft motor encoder connector is loose or fall off, restore it to normal and restart the system. 2. Check whether the motor zero point correction setting is correct; Reset the motor zero point correction; Whether there is oil on the encoder code plate, please clean it if there is any. 3. If it still does not work normally, please replace the control box or main shaft motor and notify the after-sales service. |
| E14 | Main shaft motor encoder signal is abnormal | 1. Turn off the system power, check whether the main shaft motor encoder connector is loose or fall off, restore it to normal and restart the system. 2. Check whether the grating is installed correctly (whether the grating screws are tightened and whether the grating is in the center of the encoder). 3. Check whether there is oil on the encoder code plate, if there is, please clean it up, and restart the system after recovery. 4. If it still does not work normally, please replace the control box or main shaft motor and notify the after-sales service. |
| E15 | Main shaft motor drive overcurrent | 1. Please check whether the motor power cord has bad contact. 2. Please check whether the motor power cord is crushed. 3. Please replace the control box or main shaft motor and notify the after-sales service. |
| E16 | Thread trimming position is abnormal | 1. Turn off the power of the system and check whether the thread trimming mechanism will return to the correct position. 2. Check if the setting of thread trimming protection switch detection is correct. 3. Check if the setting of thread trimming is correct. 4. If it still does not work normally, please replace the control box or panel and notify the after-sales service. |
| E17 | Positioning signal of hall components is abnormal | 1. Turn off the system power, check whether main shaft motor power cord connector and encoder connector are loose or fall off, restore them to normal and restart the system. 2. Check that the magnet on the front airfoil of the motor is installed correctly and that the magnet is not present. 3. If it still does not work normally, please replace the control box or main shaft motor and notify the after-sales service. |

4 Port diagram

14P function port description

1. Thread trimming of upper knife electromagnet: 2 (signal), 9 (+32V)

2. Thread trimming of lower knife electromagnet: 3 (signal), 10 (+32V)

3. LED light: 4 (DGND), 11 (+5V)

4. Thread trimming protection: 5 (DGND), 12 (+15V), 13 (signal)

6P function port description

Pedal: 1 (DGND), 3 (+5V), 6 (signal)

2P function port description

Presser foot lifting: 1 (signal), 2 (+32V)

2P function port description

Blowing: 1 (+32V), 2 (signal)